OS/tools Incorporated

COM/8 Asynchronous Device Driver

Support for Quatech Incorporated Adapter Boards

Installation

There is no automatic installation program for the QCOMDD.SYS

device driver supplied by Quatech.  You can use the method

described in DDRIVER.DOC for the Custom and Evaluation versions of

the device driver.

MCA Adapter Configuration

When configuring Quatech MCA adapter boards you must select the

"Scratchpad" option and NOT the "Interrupt Status" option for each

adapter board to be accessed by the QCOMDD.SYS device driver.  If

you select the "Interrupt Status" option the devices on that

adapter board will not be recognized as valid serial devices.  See

Quatech documentation for an explanation of "Scratchpad" versus

"Interrupt Status".

If you want to use the QCOMDD.SYS device driver for non-Quatech

serial ports you must have at least one Quatech Asynchronous Serial

adapter board installed in any slot in your machine.

ISA Adapter Configuration

If you have Quatech ES-100 (eight port), or QS-100 (four port)

asynchronous adapter boards, you will be able to take advantage of

some Quatech specific features built into the QCOMDD.SYS device

driver.

These features include:

1.   The capability of using more than one adapter board (QS-

100) on a single interrupt level, spanning multiple

slots.

2.   Defining interrupt identification registers for each

installed adapter board, therefore giving more dependable

and less error prone operation in the ISA edge sensitive

interrupt environment.

3.   Access to different baud rate ranges, by changing the

baud rate clock jumpers on the adapter boards and using

the explicit baud rate divisor feature of the QCOMDD.SYS

device driver (bit 11 of the "T" extension mask switch).

There is one additional command line switch and one additional bit

definition for the "T" extension mask switch that will allow you to

take advantage of Quatech ISA adapter features.

An "S" following a "/" on the "DEVICE=QCOMDD.SYS" command line

indicates to the device driver that an Interrupt Identification

Register address specification follows.  The number following the

"S" tells the device driver an I/O address at which to expect an

interrupt identification register.  Up to four "S" switches will be

recognized and each switch indicates an interrupt identification

register for one installed adapter board.

The "S" switch is to be used in conjunction with bit 14 of the "T"

extension mask switch.  Bit 14 of the extension mask must be set to

enable the Quatech Specific features of the QCOMDD.SYS device

driver.  See extension mask explanation under Device Driver

Configuration in DDRIVER.DOC.

There is one other Quatech specific switch.  The "s" switch is to

be used when you want to use the QCOMDD.SYS device driver with

non-Quatech Asynchronous Serial ports.  A "number" is to follow the

"s".  The "number" represents the DOS COMx port number of the first

QCOMDD.SYS DOS COMx device "connected" to a Quatech adapter board

port.

The "S" switch and the "s" switch are, of course, case sensitive.

To take advantage of features 1 and 2 (above) you must set bit 14

of the extension mask switch for all Quatech ports to be supported

(i.e. with a global "T" switch) and specify an interrupt

identification register (with the "S" switch) for each Quatech

adapter board to be supported.  You must also configure your

Quatech adapter boards for the "Interrupt Status" option and NOT

the "Scratchpad" option.  See Quatech documentation for details on

how to configure your adapter boards for the "Interrupt Status"

option.  When taking advantage of these features the device driver

can support only one interrupt level for each "DEVICE=QCOMDD.SYS"

line in your CONFIG.SYS file.

If you want to use the QCOMDD.SYS device driver for non-Quatech

serial ports you must:

1.   Have at least one Quatech adapter board installed in your

machine.

2.   Position the "DEVICE=QCOMDD.SYS" line to be "connected"

to the Quatech adapter(s) before any "DEVICE=QCOMDD.SYS"

line to be "connected" to a non-Quatech serial port in

your CONFIG.SYS file.

3.   Use the "s" command line switch in any non-Quatech

"DEVICE=QCOMDD.SYS" "connections" to indicate the DOS

COMx port number of the first "connected" Quatech serial

port.

Example:

DEVICE=QCOMDD.SYS /i1-10 /p1-380h /C3-6 /T1-4000h /S387h

DEVICE=QCOMDD.SYS /c1-2 /1i4p3f8h /2i3p2f8h /s3

The first line tells the first device driver load to:

1.   /i1-10 - Use interrupt level ten for all logical devices

from one through eight.

2.   /p1-380h - Use base address 380h for logical device one,

388h for logical device two, 390h for logical device

three, 398h for logical device four, 3A0h for logical

device five, 3A8h for logical device six, 3B0h for

logical device seven, and 3B8h for logical device eight.

3.   /C3-6 - Map COM3 to logical port one, COM4 to logical

port two, COM5 to logical port three, COM6 to logical

port four.  The device driver will not try to initialize

any logical port above four.

4.   /T1-4000h, bit 14 - Enable Quatech feature support for

logical ports one through eight.

5.   /S387h - Expect an interrupt identification register at

I/O address 387h.

The second line tells the second device driver load to:

1.   /C1-2 - Map COM1 to logical port one, COM2 to logical

port two.  The device driver will not try to initialize

any logical port above two.

2.   /1i4p3f8h - Assign to logical device one a base I/O

address of 3f8h and interrupt level four.

3.   /2i3p2f8h - Assign to logical device two a base I/O

address of 2f8h and interrupt level three.

4.   /s3 - Look to COM3 for verification of Quatech adapter

presence.  If COM3 had not been successfully "connected"

by a previous QCOMDD.SYS device driver load, then this

load will fail.

The first line of the preceding example could be used to "connect"

a Quatech QS-100 (four port) adapter board.  The values 387h, 38fh,

397h, or 39fh would all be valid interrupt identification register

definitions for this assumed configuration of a QS-100.  Only one

"S" switch register definition for each Quatech adapter board can

and should be used.

If you are using the evaluation version of the device driver

(COMDDE.SYS) you cannot configure the adaptor board for the

"Interrupt Status" option unless you turn on bit 14 of the

extension mask command line option ("T" switch).  You must specify

at least one interrupt identification register address ("S" switch)

if you configure your Quatech adapter for "Interrupt Status".  The

presence of a Quatech adapter board is not required when using the

evaluation version of the COM/8 device driver, and therefore, the

"s" switch will have no effect.

The Quatech supplied version of the COM/8 device driver will not

work with the Quatech DS-100 adapter boards as there is no

interrupt ID register on these boards.  You must purchase another

version of the COM/8 device driver directly from OS/tools for use

on Quatech DS-100 adapters.