OS/tools Incorporated

COM/8 Asynchronous Device Driver

Information and Installation Guide

Introduction

The OS/tools Incorporated COM/8 Device Driver for OS/2 is capable of

handling up to eight asynchronous serial devices for each COM/8

"DEVICE=" line in the CONFIG.SYS file.  It can either replace or

supplement either the COM01.SYS asynchronous device driver supplied

by IBM and Microsoft for use with Industry Standard Architecture

(ISA) machines, the COM02.SYS device driver supplied by IBM and

Microsoft for use with Micro Channel Architecture (MCA) machines and

versions 1.x of OS/2.  The COM/8 device can also replace and/or

supplement the COM.SYS device driver supplied with OS/2 2.0.

Version Description

There are three different versions of the COM/8 device driver:

1.   A standard version, COMDD.SYS, that will work with any IBM

compatible Asynchronous Serial port and/or adapter board.

2.   Custom versions, xCOMDD.SYS, that will only work with

specific adapter boards.  Where the "x" varies with the

adapter board the custom COM/8 device driver is optimized

for.  These versions are usually distributed by the OEM

suppliers of specific adapter boards.

3.   An evaluation version, COMDDE.SYS, will work on any IBM

compatible Asynchronous Serial port or adapter board, but

has some features disabled.  See Evaluation Device Driver

Restrictions under Application Interface, below.

Systems Supported

The COM/8 device driver supports two of the three platforms that support

the OS/2 operating system.  The two platforms supported are the Industry

Standard Architecture (ISA), or IBM AT bus, and the Micro Channel

Architecture (MCA) bus.  Specific support for the Extended Industry

Architecture (EISA) is expected by January of 1993.

Micro Channel Architecture Support

The COM/8 device driver will automatically configure a Micro Channel

Architecture machine with asynchronous serial ports that use base

I/O addresses and interrupt levels the same as those specified by

IBM for MCA machines.  The I/O base addresses and interrupt levels

for Asynchronous Serial Devices supported for automatic

configuration by the COM/8 device driver are:

Base I/O address      Logical Port      Interrupt Level

3F8h                  One               4

2F8h                  Two               3

3220h                 Three             3

3228h                 Four              3

4220h                 Five              3

4228h                 Six               3

5220h                 Seven             3

5228h                 Eight             3

If the device driver finds a valid Asynchronous Serial device at one

of the above I/O base addresses that is connected to the

corresponding interrupt level, then that device is initialized and

made available to applications.  Unlike the COM02.SYS or COM.SYS

device drivers supplied with OS/2 (for use with MCA machines), the

COM/8 device driver is designed to have a one-to-one relationship

with available port base addresses and DOS device names.  This means

that a DOS device name (e.g. COM1) will be mapped directly to a

corresponding hardware address (e.g. logical port one).  Ports will

be mapped as shown below.

Logical Port       DOS Device Name

One                COM1

Two                COM2

Three              COM3

Four               COM4

Five               COM5

Six                COM6

Seven              COM7

Eight              COM8

If you install devices that fill address space for port's one,

three, and four, then DOS devices COM1, COM3, and COM4 will be

available to applications after automatic configuration at system

start-up (boot).

If you need consecutive DOS device name numbering, you must

configure your hardware for consecutive base addresses as described

above, or specify the pertinent information about your hardware on

the  COM/8 device driver command line (see explanation under device

driver installation).

Industry Standard Architecture Support

There is no automatic configuration for Industry Standard

Architecture (ISA) machines.  Because ISA machines use edge

sensitive interrupts, there are restrictions the COM/8 device driver

must place on port address and interrupt level selection.

In an ISA machine, devices that have the same interrupt level must

be on the same expansion board (all in one slot).  A maximum of

eight devices can be handled by the COM/8 device driver on any

single interrupt level on ISA machines.  The  COM/8 device driver

will not prevent you from using the same interrupt level across

slots.

Hardware Requirements

It is highly recommended that any asynchronous serial device used

with OS/2 have hardware buffering capabilities (i.e. using the

16550A UART).  This is especially important when more than two or

three devices are to be supported and/or baud rates above 1200 bps

are to be used.  The  COM/8 device driver will not prevent you from

using non-hardware-buffered devices.

The COM/8 device driver will perform the following tests at

initialization to verify that any device it is expected to support

is configured correctly:

1.   Test if there is a valid UART at the requested I/O base

address.

2.   Test if the requested interrupt level is available.

3.   Test if the device is connected to the requested interrupt

level.

Any discrepancies in the above tests are reported during

initialization at start-up.  Devices that fail the above tests will

not be installed and will not, therefore, be available to

applications through this device driver load.

Device Driver Installation

Standard Version

If you have the standard version of the device driver you must

use the installation program (INSTALL.EXE) supplied on the

distribution disk.

Place your distribution disk in either drive A or drive B and

change the default drive to that drive (e.g. [C:\]A: <ENTER>).

Enter INSTALL at the prompt, followed by the drive and

directory path where you want to install the driver (e.g.

[A:\]INSTALL c:\os2 <ENTER>).

You will be prompted for a configuration string during the

installation process.  Be sure you are familiar with your

hardware configuration and what you will need to place in the

configuration string, based on that hardware configuration.

See Device Driver Configuration below for details on how to

assemble a configuration string.  A "DEVICE=COMDD.SYS

'configuration string'" line will be written, automatically,

into your CONFIG.SYS file by the installation program.

Multiple License, Custom, and Evaluation Versions

If you are installing a multiply licensed, custom (OEM) version

(xCOMDD.SYS), or the evaluation version (COMDDE.SYS) of the

device driver then copy the device driver from the distribution

disk to a directory on your boot disk and add this line to your

CONFIG.SYS file:

DEVICE=[path][driver name] [configuration switches]

Where "path" is the absolute directory path you copied the

device driver to and "driver name" is either COMDDS.SYS,

COMDDE.SYS, or xCOMDD.SYS, where "x" is dependent on which

OEM the device driver is customized for.

Includes Rev's E, Em, Q, Qm, N, Nm, S, and Sm.

Device Driver Configuration

If you have non-standard hardware, with different base addresses or

different interrupt levels than those supported for automatic

configuration, or more than eight devices on an MCA machine, or you

have an ISA machine, you will have to define the device driver

configuration with command line switches.

All switches begin with "/" or "-".  The first character after the

"/" or "-" will be a "V", a number from "1" to "8", an "I", a "P",

an "M", a "T", a "C", or a "D".

A "V" following a "/" enables verbose mode and tells the driver to

write to the screen, at start-up time, the base I/O address,

interrupt level, and non default buffer sizes for each logical

device the driver controls.  A "D" following the "V" will cause the

device driver to delay after writing the "verbose" information to

the screen.  If a number follows the "D", it will be used to

determine the time (in tenths of a second) to delay after all

devices to be supported are initialized.  The default delay value is

20 (2 seconds).  The "V" switch also causes device driver version

information to be displayed during initialization.

A number "1" through "8" following a "/" represents the specific

logical device number subsequent characters in the switch refer to.

A"1" following the "/" would refer to the first logical device.

There are up to eight logical devices for each COM/8 "DEVICE=" line

in the CONFIG.SYS file.  After the "number" the following characters

have meaning:

"I"  Tells the driver that the following number is the

interrupt level to use for this logical device.  All

interrupt levels except 0 and 1 are supported.

"P"  Tells the driver that the following number is the base I/O

address of this logical device.

"M"  Precedes a buffer size specification for this logical

device; an "MR" indicates a read buffer size follows, an

"MW" indicates a write buffer size follows, and an "MI"

indicates a COMscope buffer size follows.  Any "MI" switch

also enables COMscope extentions for the device driver.

"D"  Precedes a COM default specification

Position 1      2     3     4

D      T     R     #

D      T     W     #

D      B     #

D      F     1     #

D      F     2     #

D      F     3     #

D      L     #

D      I

"T"  Precedes an extension mask for this logical device.

NOTE:     No white space or delimiters are required or allowed

within a logical device specific switch as described

above.

An "I" following a "/" will indicate a global interrupt level

definition follows.  The number following the "I" tells the device

driver the logical device number to start applying the interrupt

level definition to.  A delimiter ("-", ",", or single space) and a

number follow.  The number is to indicate the desired interrupt

level to use for all of the devices from the specified starting

logical device to logical device number eight.  Again, all interrupt

levels except 0 and 1 can be selected.

A "P" following the "/" indicates a global starting consecutive port

base address follows.  The number following the "P" tells the driver

the logical device number to start applying the starting consecutive

base address to.  A delimiter ("-", ",", or single space) and a

number to indicate the I/O base address to apply to the starting

logical device follow.  All succeeding ports (up to and including

logical device eight) are then given base addresses by adding eight

to the base address applied to the previous port.

An "M" following the "/" indicates a global buffer size

specification follows.  Either an "R" (for Read), a "W" (for Write),

or an "I" (for COMscope) following the "M" will indicate which

buffer the global switch refers to.  Immediately following either

the "R", "W", or "I" is a number to indicate starting logical device

number to apply the global buffer specification to.  After a

delimiter ("-", ",", or single space) a number to indicate the

requested size of the specified buffer follows.    "MI" also enables

the COMscope extentions for the device driver.  If no number follows

the "MI" switch a default buffer size of 64 bytes will be allocated.

A "D" following the "/" indicates a global COM default specification

follows.  A "T" (for Tmeout), a "B" (for Baud rate),  an "F" (for

DCB Flag), an "L" (for Line characteristics), or an "I" (for IBM

defaults) is to follow the "D".  A "T" will be followed by either an

"R" (for  Read) or a "W" (for Write).  An "F" will be followed by a

number from 1 to 3 to indicate the DCB flag to set defaults for (see

the "Physical Device Driver Technical Reference", Category 1, I/O

control function 53h for a description of these flags).  An "I"

following the "D" will cause that logical port to use the defaults

suggested by IBM for Asynchronous device support for that logical

port.  A number after the "R" or "W" after "T", the "B", the flag

number, or the "L" tells the device driver the logical device number

to start applying the default specification to.  After a delimiter

("-", ",", or single space) the second number will represent the

value to set the default to.

Position 1     2     3      4     5     6

D     T     R      #     -     #

D     T     W      #     -     #

D     B     #      -     #

D     F     1      #     -     #

D     F     2      #     -     #

D     F     3      #     -     #

D     L     #      -     #

D     I     #

First number (#) is logical device to start default application.

Second number (#) is value to set default to.

Default Line Characteristic bit assignments:

Function             Bit 7  Bit 6  Bit 5 Bit 4  Bit   Bit 2  Bit   Bit 0

3            1

Data bits                                                      X     X

Stop Bits                                               X

Parity Enable                                     X

Even Parity Select                         X

Stick Parity Select                  X

Data Bits  Bit 1  Bit 0

5         0      0

6         0      1

7         1      0

8         1      1

Stop Bits               Bit 2

1                         0

1.5 (with 5 data bits)    1

2 (with 8 data bits)      1

Default Line Characteristic Examples:

8 data bits, no parity , 1 stop bit -> use:    /DL1-03h or /1DL03

7 data bits, even parity, 1 stop bit -> use: /DL1-1Ah or /1DL1Ah

For timeouts ("T") the vaule will indicate a default timeout, in .01

second increments, for its respective flow direction (Read or Write)

for that logical device.  For baud rate ("B") the value will

indicate the required default baud rate for that logical device.

For line characteristics ("L") the value will represent the default

line characteristics required for the logical device.  For DCB flags

the value will represent the default value to set the flag to.

"T" following the "/" indicates that a Device Driver extension mask

follows.  The number following the "T" tells the device driver the

logical device number to start applying the extension mask to.

After a delimiter ("-", ",", or single space) the second number will

enable certain extensions as described below:

Bit  Function

0    reserved (must be zero).

1    Disable Modem Interrupts (Modem Signals will still work)

2    reserved (must be zero).

3    reserved (must be zero).

4    Disable initialization hardware tests.

5    reserved (must be zero).

6    reserved (must be zero).

7    reserved (must be zero).

8    reserved (must be zero).

9    reserved (must be zero).

10   Enable access to OUT1 and LOOP modem control bits.

11   Use requested baud rate as explicit UART clock divisor.

12   reserved (must be zero).

13   reserved (must be zero).

14   reserved (must be zero).

15   Print System Location Information at initialization.

A "C" following the "/" indicates a DOS COMx port range map follows.

The number following the "C" tells the driver the DOS port number to

start mapping the device driver's logical ports to.  After a

delimiter ("-", ",", or single space) the second number indicates

the last DOS port number to be handled by the COM/8 device driver.

Up to eight devices can be supported by the COM/8 device driver for

each COM/8 "DEVICE=" line in the CONFIG.SYS file.  DOS COMx port

numbering will be consecutive, starting with the first number

indicated and ending with the second number indicated.

The COM/8 device driver command line interface is NOT case sensitive

("i" = "I").  Any numerical value can be in either base 10 or base

16 (hexidecimal).  An "H" must directly follow a base 16 number

(e.g. 3f8h).  All switches for the device driver being loaded must

be on the same line as the COM/8 "DEVICE=" in the config.sys file.

Once a particular configuration parameter for a logical device has

been specified by some command line switch, a following conflicting

global parameter specification switch will be ignored.  This is

useful if you want to specify two or more, different base I/O

address sequences, or interrupt levels, globally.  Just specify the

global switch for the higher logical port numbers first.  The last

port specific parameter specification will always override any

global or previous port specific specification.  We are just

specifically specifying some particular specificity about very

specific items, globally speaking (writing) that is, in case you

hadn't noticed.  You may ignore that last sentence, if you can.  It

was getting late.

Configuration Example:

/VD100 /1p2000hi4mw800hb4800 /2F343hL03TR9B38400 /i2-10 /p2-

380h /C2-6 /T1-8000h /b3-2400 /DI1

This example tells the device driver to:

1.   /VD100 - Be verbose and delay ten seconds after writing

initialization and version messages to the screen and

initializing all ports to be supported by this device

driver load.

2.   /1p2000hi4mw800hb4800 - Assign to logical device one a

base I/O address of 2000h, interrupt level four, set write

buffer length to 2048 bytes, and to set its baud rate to

4800 bps.

3.   /2F343hL03TR10B38400 - Assign to logical device two a DCB

flag3 enabling both transmit and receive Xon/Xoff flow

control (handshaking) with full duplex on, Line

characteristics of 8 data bits, one stop bit, and no

parity, a read timeout of 100 milliseconds, and a baud

rate of 38400 bps.

4.   /i2-10 - Use interrupt level ten for all logical devices

from two through eight.

5.   /p2-380h - Use base address 380h for logical device two,

388h for logical device three, 390h for logical device

four, 398h for logical device five, 3A0h for logical

device six, 3A8h for logical device seven, and 3B0h for

logical device eight.

6.   /C2-6 - Map COM2 to logical port one, COM3 to logical port

two, COM4 to logical port three, COM5 to logical port

four, and COM6 to logical port five.  The "C" switch

defines and limits the range of DOS COMx ports handled by

the device driver load module.

7.   /T1-8000h, bit 15 - Print device driver memory location at

initialization.

8.   /b3-2400 - Assign to logical devices 3 through 8 a default

baud rate of 2400 bps.

10.  /di1 - Assing to all logical devices the defaults

suggested by IBM in the "Physical Device Driver Technical

Reference" for Asynchronous communications devices.  NOTE:

those parameters previously will not be changed,  only

those that have not been assigned will be set to the IBM

defaults.

The maximum buffer space for each device driver load module is about

63k bytes, total.  You must divide the available space to suit your

needs.    The default receive buffer size is 2048 bytes and the

default write buffer size is 256 bytes.  The minimum buffer size is

128 bytes.

Using the COM/8 command line switches will override any automatic

configuration expectations.  A switch specified interrupt level will

take affect in any case, as will explicit buffer sizes.  A port

specific switch will always override a global switch.

If left alone (no command line switches to the contrary), the

defaults for the COM/8 device driver are:

Parameter                       Default Values

Read timeout                    normal, 100 milliseconds

Write timeout                   normal, 1 second

Line characteristics            8 data, no parity, 1 stop

RTS                             enabled  (no handshaking)

DTR                             enabled  (no Handshaking)

CTS                             ignored

DSR                             ignored

DCD                             ignored

FIFOs                           automatic (if present)

Xon/Xoff, transmit and receive  disabled (no handshaking)

All other parameters are as suggested by IBM.  Default baud rates

from 2 to 57600 bps can be specified on the COM/8 device driver

command line.

If you specify a base I/O address of FFFFh (65535) for a logical

device, the logical device number and the DOS COMx device name

mapped to that port will not be accessed or controlled by this

device driver load, and therefore, may not be accessible to

applications.

If you enable System Location Information (bit 15 of the "T" switch)

you will see a message like the following at the end of all other

information printed at initialization.

0309:2230 - 3680 C

0319:0380 - 0482 - 3662 D

The first line of the above System Location messages indicates that

the code segment selector is 0309h (CS), that the run-time code

segment limit is 2230h (CSLIM), and that the initialization-time

code segment limit is 3680h (CSLIM).  The second line indicates that

the data segment selector is 0319h (DS and/or ES), that the run-time

driver variable area length is 0380h, that the initialization-time

driver variable area length is 0482h, and that the run-time data

segment limit is 3662h (DSLIM and/or ESLIM).

It is recommended that you use the Verbose-Delay (e.g. /VD100)

switch to give you time to read and/or copy the System Location

information.

Application Interface

The application interface for all ports under COM/8 control is

exactly the same as for the COM02.SYS, or COM01.SYS device drivers

supplied with OS/2 1.x and the COM.SYS device driver supplied with

OS/2 2.0.  You must issue a DosOpen and use the handle returned by

that function to issue DosWrite, DosRead, and DosDevIOCtl requests.

See the OS/2 "Physical Device Driver Technical Reference"

publications under "DosDevIOCtl" for details on using I/O control

functions for the COM/8 device driver.  The standard and OEM custom

device drivers support all of the features described in OS/2

technical publications for Asynchronous Serial devices (Category 1).

All versions of the COM/8 device driver are designed to operate the

same as the COM02.SYS and COM01.SYS device drivers supplied with

OS/2 1.x and the COM.SYS devices driver supplied with OS/2 2.0, with

a few exceptions:

1.   DOS COMx device name is based solely on base I/O address

and/or explicit command line switches.

2.   Up to eight COMx devices are supported for each COM/8

"DEVICE=" line in your CONFIG.SYS file.

3.   Up to thirteen different hardware interrupt levels are

supported.

4.   Baud rates up to 115.2k, and above, if hardware is

capable.

5.   The default receive trigger level on ports with 16550A

UARTs is 14 bytes.

6.   Ports can be accessed from a DOS window only through the

DOS INT 21h file system interface.

7.   The MODE command will only work with COM1 through COM9.

This is a limitation of the current MODE command

asynchronous port support supplied with OS/2.  Use

COMscope or command line switches to set COM ports above

COM9.

8.   The device I/O control functions 43h and 63h can be used

to set and query baud rates above 57.6K.  This is also

possible with the COM.SYS device driver supplied with OS/2

2.0.

Evaluation Device Driver Restrictions

The COMDDE.SYS device driver has been partially disabled.  When

the device driver is first initialized at start-up, only 1000

DosWrite requests will be processed before write capabilities

will be disabled.  Once write capabilities have been disabled

they can be restarted again by first closing the port (issuing

a first level DosClose request), then re-opening the port

(issuing a first level DosOpen request).  Each re-open will

then allow half the number of write requests as the previous

close/open sequence allowed, until no write requests, at all,

are allowed.  At this time all DosRead requests will be denied

also.  You will then have to re-boot your system to re-

initialize the device driver.

COMscope Communications Monitor

In order to use the COMscope Communications Monitor, you must

include at least one "MI" switch on the device driver command line.

Just add something like "/MI1-256" or "/1MI100h" to the COM/8

"DEVICE=" line in your CONFIG.SYS file to enable support for

COMscope.EXE.  In addition the COM/8 device driver must have a lower

case "m" in its Revision number, as displayed at system

initialization, for COMscope to work with it (e.g. Revision Sm).

Setting the "MI" option will allow COMscope to non-invasively

monitor the data stream and states of the device driver and hardware

for any, or all, of the devices controlled by the COM/8 device

driver.

When you select the Device Driver Status option, under the Monitor

menu heading, a movable window will be placed on screen that

displays the various status words available from the device driver.

If you click the mouse pointer on any of the values displayed,

another window will open.  That window will list the meanings of

each bit in the status word and show their states at the time the

mouse was clicked.

Unless you enable the Stream Monitor Function from the "Monitor"

menu, any key pressed while the main window is active will be sent

to the device driver via the Transmit Byte Immediate I/O control

function (0x44h).

You can have an application that accesses a COM port, and COMscope,

monitoring that same port simultaneously.  COMscope can be loaded

multiple times to monitor multiple COM ports.

If you select "Count Characters" from the "Options" menu you can

calculate the number of character positions from the position of the

click of the left mouse button to the position of the click of the

right mouse button.

The monitor display consists of alternate white on black and black

on white lines.  When a character is read from the device hardware

it will be displayed in the white on black line (odd lines).  When a

character is written to the device hardware it will be displayed in

the black on white line (even lines).

The data stream collected by COMscope is continually written to the

display buffer as characters are sent to and received from the

device hardware.  When the display buffer is full the data stream

will wrap around to the beginning of the buffer.  The display buffer

can hold around 15,000 characters.

The size of the buffer specified on the device driver command line

should be selected according to the baud rate at which you expect to

operate the device.  A buffer size of 256 (e.g. /1MI100h) may be too

small for 9600 baud; and that's all the help you get.

The bottom line of the display window will display COMscope errors

when "Error Display" is selected from the "Options" menu (assuming

there are errors to display).  Selecting "Count Characters from the

"Options" menu lets you count characters in the display buffer.

When neither the "Count Characters" or the "Error Display" items are

enabled, the bottom line will display the position in the display

buffer of the top of the window (TOW) and the current position (CUR)

of the next character to be written to the display buffer.  An up or

down arrow will show the relative position of those two lines, and a

block will be displayed whenever the next character to be written

line is within the COMscope display window.

COMscope monitors the character stream between the hardware and the

device driver.  We have another product that can be used to monitor

the data stream between two different serial ports.

The version of the COMscope Monitor that will work with the

evaluation version of the device driver is supplied as MIRROR.EXE

when it is necessary to supply COMscope version for both the

evaluation and normal versions of the device driver.  The version of

COMscope for the evaluation version of the COM/8 device driver can

be distributed freely, along with the evauation version of the

device driver itself.

Advertisement

We have attempted to produce products that meet the communication

needs of our current customer base.  If you or your company have

special needs, please feel free to contact us about those needs.

Our address is:

OS/tools Incorporated

124H Blossom Hill Road

Suite 1522

San Jose, CA USA 95123

You may phone or FAX us at (408)225-7188.

Credits

IBM is a trademark of International Business Machines Incorporated.

Microsoft is a trademark of Microsoft Corporation.  OS/2 is a

trademark of IBM and Microsoft.  Micro Channel Architecture is

trademark of IBM.  OS/tools is a trademark of OS/tools Incorporated.

License

You may copy and distribute the evaluation version (COMDDE.SYS)

freely.  You may not modify any version of the COM/8 device driver

or its utilities in any way.  We own it and you may use it on a one

license per CPU license basis.

Disclaimers

The usual warranties and subsequent disclaimers, of course, apply.

We know nothing and are responsible for nothing.  Thank you for your

support.